Preparation of palynological slides

- Chemical processing

- grounding of dry weighted sediment to favour the chemical leaching. The samples are placed in plastic pots and labelled;

- leaching with cold HCl (35%) to remove carbonates. After effervescence, pots are filled with water for decantation (24 hours). Floating residues and water in excess are removed and the underlying carbonate-free sediment carefully retained at the bottom;

- two successive leaching with cold HF (70%) are applied to remove silicates, with a time interval of 6 hours in between. Proceedings are followed by washing and decantation for 24 hours;

- residual sediment is transposed into preliminary labelled tubes bearing a conical base. They are washed with HCl and water, respectively, using three successive centrifugations (5 min at 2500 runs per min) to remove leached residuals;

- residuals are treated with ZnCl2 (density = 2.0) and centrifuged for 10 min (at 1000 runs per min) to separate denser particles accumulating in the conical base of the tube from the overlying floating residue containing palynomorphs. The floating residue is further diluted with HCl (35%), centrifuged and washed. None palynomorph should be lost in the denser residue;

- for sediments extremely rich in vegetal remains, a further treatment with KOH (10%) is applied to remove them;

- residues containing palynomorphs are successively sieved using 150 µm- and 10 µm-nylon meshes to remove (1) the coarser particles including organic macroremains, and (2) clays after a brief sonication (about 30 s). Follows a centrifugation to concentrate palynomorphs in the conical base;

The following step is facultative and concerns only dinoflagellate cysts. At that stage, the palynological residue can be stained using safranin-O. This method usually allows identifying reworked specimens (see Chapter IV.2.4).

- The tube is reversed and let for air-drying during about one hour. The final residue is then diluted at 1/10 with glycerol. Using a micropipette, 50 µl are mounted onto palynological slides preliminary labelled and the coverslips are sealed with LMR histological glue. The palynological residue is finally stored in small archive plastic tubes preliminary labelled.

- For scanning electron microscope analysis, residues must be sieved again to remove the glycerol, washed with distilled water and air-dried onto small circular metal blocks for about two hours, mounted onto metal stubs, and sputter-coated with gold.

- Calculation of the richness in palynomorphs

The fraction of the palynological residue is calculated as follows, according to Cour (1974):

F (%) = v/V * 1/L *100

where v is the volume of residue placed upon the slide (preliminary measured: 50 µl), V is the total volume of palynological residue, L is the mean width of the coverslip and l the diameter of the examined field multiplicated by the number of lines analysed.

When F is known, the concentration (richness) in palynomorphs (pollen grains, dinoflagellates cysts) can be calculated using the following formula (Cour, 1974):

r (richness) = (100*n) / (F*m)

where n represented the total number of palynomorphs counted, F is the fraction of residue analysed and m is the mass of sediment processed. Data on absolute and relative abundance, as well as concentration calculations, are presented in the CD-ROM joined to this thesis.

The concentration curve helps to verify the reliability of the calculated relative abundance, i.e., to examine whether relative abundance are biased by concentration values, themselves linked to dilution during sedimentation processes (the higher the sedimentation rates, the higher the dilution) and productivity (not examined here). For that purpose, a PCA analysis performed on selected variables such as the relative abundance and concentration of taxa was applied using the software Past. The results revealed that no link exists between the different variables, therefore allocating high confidence to our data.

- Identification and counting

Pollen grains have distinct shapes and surface textures that are primarily used to define the taxonomy of pollen grains. The number of apertures (from none to a high number) within ectexine and/or endexine, as well as their structure and shape (pores, furrows and pores associated) combined with the shape of the pollen itself (length, width-), the structure of the membranes and the ornamentation of the exine, constitute so many juxtaposed criteria that define and governs pollen grain identification (Erdtmann, 1966). For observation and identification under the photonic microscope, the L.O. analysis approach (Erdtmann, 1966) has been used. This method consists in the understanding of the pollen morphology based on a series of optical observation at different levels through the membranes. Such a detailed examination, from the upper surface of the ectexine to the lower layers of the endexine, provides a progressive 3-dimensional picture of the pollen grain structure, and enables its identification as representative of a plant in comparison with modern pollen photographs. Pollen grains of trees and shrubs are generally identified to the genus level, sometimes at the species one; those of herbs usually at the family level, sometimes at the genus one, scarcely at the species one. The ‘Laboratoire PaléoEnvironnements et PaléobioSphère’ (Lyon) benefits from a rich collection of pollen photographs and atlases, as well as its pollen database “Photopal” (http://medias.obs-mip.fr/photopal).